Global Handset Requirements for CDMA — USSD                                              (Unstructured Supplemental Service Data)

CDG Document 205

Version 1.0

10 Dec 2012

CDMA Development Group

575 Anton Boulevard, Suite 440

Costa Mesa, California 92626

PHONE +1 888 800-CDMA

+1 714 545-5211

FAX +1 714 545-4601

http://www.cdg.org

cdg@cdg.org

Notice

Each CDG member acknowledges that CDG does not review the disclosures or contributions of any CDG member nor does CDG verify the status of the ownership of any of the intellectual property rights associated with any such disclosures or contributions. Accordingly, each CDG member should consider all disclosures and contributions as being made solely on an as-is basis. If any CDG member makes any use of any disclosure or contribution, then such use is at such CDG member's sole risk. Each CDG member agrees that CDG shall not be liable to any person or entity (including any CDG member) arising out of any use of any disclosure or contribution, including any liability arising out of infringement of intellectual property rights.

- Contents

Global Handset Requirements for CDMA — USSD  (Unstructured Supplemental Service Data)	i

CDG Document 205	i

Version 1.0	i

10 Dec 2012	i

1. Introduction	1

1.1 Scope of Document	1

1.2 USSD description	1

1.3 Organization	2

1.4 USSD Use Cases	2

1.5 Reference Documents	2

1.6 Acronyms and Abbreviations	3

1.7 Terms and Definitions	3

1.8 Carrier Acceptance	4

1.8.1 Documentation	4

2. USSD Requirements	5

2.1 USSD messaging protocol requirements	5

2.2 USSD User Interface requirements	5

2.3 USSD Storage requirements	8

Revision History

| Date | Version | Description |
| --- | --- | --- |
| 15 April 2011 | 0.01 | Preliminary draft. |
| 5 Mar 2011 | 0.9 | Update from 3GPP2 standards |
| 4 Dec 2012 | 0.91 | Updated with comments from Tata, and GHRC conference calls |
| 10 Dec 2012 | 1.0 | Reviewed Accepted and Released – GHRC Meeting in Maui |

This page intentionally left blank.

# Introduction

## Scope of Document

The objective of this document is to provide requirements to USSD (Unstructured Supplementary Services Data) in CDMA handsets. This document specifies signaling requirements to support USSD, handset user interface behavior, and storage requirements. Additionally, user interface for devices with touch screen UI, and jog dial are also specified.

USSD is a text based communication between a handset and USSD application servers that allows for interactive session. The network provides session continuity and is very fast compared to SMS as there is no store and forward.

## USSD description

The Unstructured Supplementary Service Data (USSD) mechanism allows a Mobile Station (MS) and the Public Land Mobile Network (PLMN) operator defined application servers to communicate.

The following figure shows the handling of the USSD transactions. The MSC retrieves the USSD information as part of the subscriber profile at the time of MS registration. The USSD transactions are carried between the USSD client in the MS and the USSD GW via the MSC.

- Figure 1: USSD Overview

## Organization

This document is organized in sections relating to major functional elements:

- USSD messaging protocol requirements

- USSD User interface requirements

- USSD Storage requirements

## USSD Use Cases

The purpose of this section is to define generic use cases that will be used as the basis for deriving requirements in this document. The use cases are categorized to highlight the differences in how USSD messages are initiated either by the mobile or the network, and how the session is managed at the mobile, specifically the user interface of the mobile. The intent of this section is not to highlight or showcase different services that a carrier could provide using USSD.

- Network originated USSD message: This use case covers USSD messages that originate from the network. This particular use case covers only single messages from USSD server that does not require a response from the mobile.

- Network originated USSD session: This use case covers USSD sessions that originate from the network.

- Mobile originated USSD message: This use case covers all USSD messages that originate in the mobile. These include USSD message initiated by direct dialing of USSD digits and by invoking an USSD session via a graphic interface. The graphic interface could either be in the form of USSD browser specifically designed for USSD interactions or in the form of special characters in SMS or web browser.

- From a signaling perspective, this is viewed as Mobile originated USSD message followed by Network originated USSD session.

## Reference Documents

3GPP2 reference documents can be found at http://www.3gpp2.org/Public_html/specs/index.cfm.

CDG reference documents can be found at http://www.cdg.org.

CCF reference documents can be found at http://www.globalccf.org.

| Standard | Description |
| --- | --- |
| C.S0015-A | Short Message Services for Wideband Spread Spectrum Systems |
| X.S0065 | 3GPP2 USSD Specification |
| C.R1001 | Administration of Parameter Value Assignments for cdma2000 Spread Spectrum Standards |
| A.S0013 | 3GPP2 Interoperability Specification (IOS) for cdma2000 Access Network Interfaces |
| A.S0014 | Interoperability Specification (IOS) for cdma2000 Access Network Interfaces |
| C.S0105 | Unstructured Supplementary Service Data (USSD) Service Options for Spread Spectrum Systems |

## Acronyms and Abbreviations

Table 1-1: Acronyms and Abbreviations

| Acronym / Abbreviation | Description |
| --- | --- |
| USSD | Unstructured Supplementary Services Data |
| CDG | CDMA Development Group |
| CDMA | Code Division Multiple Access |
| Cdma2000® | TIA/EIA/IS-2000, with a 1.2288 MHz spreading rate |
| MS | Mobile Station |
| PLMN | Public Land Mobile Network |
| SMS | Short Message Service |
| WAP | Wireless Application Protocol |

## Terms and Definitions

Four categories of requirements are established:

| (M) Mandatory | The handset must support that characteristic in order to achieve approval. |
| --- | --- |
| (HD) Highly Desirable | It is highly desirable and recommended that the handset supports this characteristic. This feature may become Mandatory in subsequent versions of the document. Supporting this characteristic will be valued in the commercial promotion of the terminal. |
| (O) Optional | It is left up to the manufacturer whether or not the terminal supports this characteristic. The handset may support this characteristic. |

## Carrier Acceptance

The documentation and equipment that shall be delivered to the CDMA2000® 1x Operator for technical evaluation is detailed below.

### Documentation

| Req. # | Requirement | Category | Remarks | References | PRI Configurable |
| --- | --- | --- | --- | --- | --- |
| 1.8.1.1 | GHRC compliance report detailing compliance or non-compliance to each of the relevant sections of this document | M |  |  |  |
| 1.8.1.2 | CCF Test Report | M | Indicating Pass/Fail/NA to each of the stated requirements. If CCF Test Report is not available, an equivalent test as requested by the Operator shall be provided |  |  |

# USSD Requirements

## USSD messaging protocol requirements

The following requirements apply to parsing of USSD messages. The requirements below are based on USSD specification from 3GPP2.

| Req. # | Requirement | Category | Remarks | References | PRI Configurable |
| --- | --- | --- | --- | --- | --- |
|  | The handset shall support USSD DBM “001001” | M | New DBM for USSD support in CDMA2000 | X.S0065 |  |
| 2.1.2 | The handset shall support SO78 for rate set 1 | M | Rate set 1 service option | C.S0105 |  |
| 2.1.3 | The handset shall support SO79 for rate set 2 | M | Rate set 2 service option | C.S0105 |  |

## USSD User Interface requirements

The requirements in the user interface section are based on typical USSD use cases.

| Req. # | Requirement | Category | Remarks | References | PRI Configurable |
| --- | --- | --- | --- | --- | --- |
|  | The handset shall support USSD browser | M |  |  |  |
| 2.2.2 | USSD browser shall be a full screen browser | M | Full screen shall be used instead of partial screen or “pop up”. |  |  |
| 2.2.3 | USSD browser shall support static logo display | HD | This is usually reserved for Operator brand logo, and will be statically loaded on to the browser. This will not be updated over the air. This will be a PRI configurable item. |  | Yes |
| 2.2.4 | The handset shall identify USSD messages based on first and last character of dialed digits | M | A call is identified as USSD call based on first and last character |  |  |
| 2.2.5 | The handset shall tag USSD digits in SMS and other textual displays | M | The USSD digits will be identified and highlighted. |  |  |
| 2.2.6 | The handset shall allow USSD message initiation from any screen that has USSD digits tagged | M | This shall be performed by selecting the highlighted digits |  |  |
| 2.2.7 | The handset shall allow for native UI to support USSD message initiation from drop down menu with text reference | M | The drop down menu shall have text reference based on carriers input. When selected, pre-stored USSD digits will be dialed out |  |  |
| 2.2.8 | USSD messages shall be displayed in the USSD browser as formatted by the USSD Application server | M | The message formatting from USSD application server shall be retained |  |  |
| 2.2.9 | USSD messages shall display the sender ID of the USSD Application gateway in the browser | M | This is mandatory requirement for some governments. The sender ID is contained in the header of USSD messages |  |  |
| 2.2.10 | USSD browser shall support USSD responses using codes | M | End user should respond to USSD messages by just pressing number key or selecting a line on the USSD message using any input mechanism. End user is not expected to press “Send” after numeral input |  |  |
| 2.2.11 | USSD browser shall provide “Options” button | M | Selecting this button should provide sub menus. The options button will be on the lower left corner of the browser |  |  |
| 2.2.12 | “Options” button selection shall display sub-buttons under “Options” | M |  |  |  |
| 2.2.13 | USSD browser shall provide “Save” button under options menu | M | The “Save” button will be under options menu |  |  |
| 2.2.14 | “Save” button selection shall save the message in SMS/Messages inbox | M | Storage requirements defined in storage section |  |  |
| 2.2.15 | “Save” button selection shall save message, and notify user with “Message Saved” on the display | M | Once message is successfully stored |  |  |
| 2.2.16 | “Save” button selection shall exit USSD session, after “Message Saved” is displayed | M |  |  |  |
| 2.2.17 | USSD browser shall provide “Forward” button under options menu | M |  |  |  |
| 2.2.18 | “Forward” button shall use SMS for forwarding USSD messages | M | Exit USSD browser, and launch SMS forward screen |  |  |
| 2.2.19 | “Forward” button shall use SMS forward screen, with contents of USSD message as contents of SMS message | M |  |  |  |
| 2.2.20 | USSD browser shall provide “Back” button under options menu | M | This button allows end user to go back to the previous items in the USSD session. However, the handset shall not save previous sessions. The previous items shall be resent from the USSD application server. |  |  |
| 2.2.21 | USSD browser shall provide “Exit” button | M | “Exit” button to be accommodated in the display screen in the right corner. Selecting this button should exit USSD browser and return the display to previous screen |  |  |
| 2.2.22 | USSD session shall be terminated once “Exit” button is selected | M | The USSD session will be immediately terminated by the handset when “Exit” button is selected |  |  |
| 2.2.23 | USSD browser shall exit if “End” key is selected at any point during the session | M | The display should return to previous screen and previous application |  |  |
| 2.2.24 | USSD session shall be terminated once “End” key is selected | M | The USSD session will be immediately terminated by the handset when “End” key is selected |  |  |
| 2.2.25 | Touch screen support | M | USSD browser shall support touch screen UI. |  |  |
| 2.2.26 | Jog dial support | M | USSD browser shall support Jog dial input. |  |  |
| 2.2.27 | All input methods supported by handset | M | Any input mechanism used in the handset should support USSD browser |  |  |

## USSD Storage requirements

| Req. # | Requirement | Category | Remarks | References | PRI Configurable |
| --- | --- | --- | --- | --- | --- |
|  | If message type is USSD, the message shall not be stored by the handset by default | M |  |  |  |
| 2.3.2 | USSD messages shall be stored in the SMS/Message Inbox, if user manually selects “Save Message” | M | If end user selects “Save Message”, the message shall be saved in SMS Inbox with time stamp, and no sender address. |  |  |
| 2.3.3 | If USSD message is saved, sender address to be filled as USSD application server ID | M |  |  |  |
| 2.3.4 | When USSD message is stored, it shall be stored as “Read” message | M |  |  |  |