### IFAST DOCUMENT						IRMG Version 2.0

I N T E R N A T I O N A L

R O A M I N G

MOBILE IDENTIFICATION NUMBER (M I N)

(IRM)

A S S I G N M E N T

G U I D E L I N E S

A N D

P R O C E D U R E S

Version: 2.0

January 2001

TABLE OF CONTENTS

1	PURPOSE AND SCOPE	3

2	IRM FORMAT AND FUNCTION	4

3	ASSUMPTIONS AND CONSTRAINTS	4

4	ASSIGNMENT PRINCIPLES	5

5	CRITERIA FOR IRM Network Code ASSIGNMENT	6

6	RESPONSIBILITIES OF IRM Network Code APPLICANTS AND ASSIGNEES	6

8	MIN RETURN AND RECLAMATION PROCEDURES	Error! Bookmark not defined.

9	MAINTENANCE OF GUIDELINES	7

11	CONFLICT RESOLUTION	7

10	APPEALS PROCESS	8

# 1	PURPOSE AND SCOPE

This document contains the guidelines and procedures for the assignment and use of International Roaming Mobile identification numbers (IRM) in ANSI-41 based mobile systems.  IRMs perform, internationally, the same function as the Mobile Identiification Number (MIN) in domestic networks. The use of IRMs is a temporary solution for international roaming between or into ANSI-41 based systems. The long term solution is the evolution to and use of the International Mobile Subscriber Identifier (IMSI), a 15-digit identifier defined in the International Telecommunications Union’s (ITU) Recommendation E.212.

Traditionally, the MIN is a 10-digit number that has been identical to the domestic Mobile Directory Number (MDN) and is defined in ANSI/EIA/TIA-553.  The MIN is used by home and visited wireless networks to identify the subscriber and to perform subscriber and service verification procedures. This use of a domestic number, if utilized across national boundaries when roaming internationally, can often  result in conflicts with MINs in  other countries, thereby, hindering or complicating international ANSI-41 based roaming.  There has been no centralized international administration for MINs and their formats. TSB-29, by listing all known MIN assignments (NANP and non-NANP) has provided a limited source for assignment coordination. The International Forum on ANSI-41 Standards Technologies (IFAST), has assumed an active role in addressing this problem by creating and administering IRMs.

  - The IRM administrator is appointed by the IFAST and manages and coordinates the assignment of the IRMs, under the oversight of the IFAST.

  - These guidelines were developed by the consensus of representatives of entities within the ANSI-41 based international wireless sector and international wireless networks in which intersystem operations for roaming and handoff are based on versions of EIA/TIA Interim Standard 41, as represented at the IFAST.

  - These guidelines apply internationally for carriers using ANSI-41 based mobile systems and/or for carriers roaming into ANSI-41 based systems from other wireless network technologies.  These guidelines do not supersede the regulations, procedures, or requirements of any appropriate national or international legal or regulatory authority.

  - Additional information may be found on the IFAST web site;  www.ifast.org

# 2	IRM FORMAT AND FUNCTION

2.1	In many ANSI-41 based countries, e.g., those countries served by the NANP, numbering resources can not have a “0” and “1” as the first digit of an MDN and, consequently, of a legitimate MIN. The IFAST determined that this non-utilized numbering plan capacity was appropriate for IRMs and determined to use this resource.

2.2	Therefore, the IRM is a valid MIN constrained to having a "0" or "1" as the first digit and having a 4-digit format.  Thereby, the IRM format is “0XXX” and “1XXX” – creating 2,000 IRMs.

  - MIN Blocks of the format 0XXX and 1XXX  are reserved to be assigned by the IFAST as IRMs, although some of these MINs have been assigned, by service providers, for specific services in the NANP-serving area in the past.  These past service assignments will continue to be valid in the NANP-serving area, and are documented.

  - The IRM, when transmitted by a roaming terminal to a visited network, is utilized by the VLR of that net work to identify the home network of the roaming subscriber in order to query that network to determine the validity of the subscriber and to ascertain billing and services information.

  - The digits following the IRM uniquely identify a subscriber of the network identiied by the IRM and are in the format appropriate for the domestic numbering plan of the home network. The IRM assignee is responsible for the assignment and management of the digits following the IRM.

# 3	ASSUMPTIONS AND CONSTRAINTS

These guidelines are based on the following assumptions and constraints:

3.1	These guidelines and procedures should provide the greatest latitude to those providing ANSI-41 based mobile systems, as well as other wireless technologies carriers providing roaming into ANSI-41 based systems, while permitting the effective and efficient management of a finite resource.

3.2	The function of the IRM administrator will be performed by a person(s) or organization so agreed to by the IFAST.  The IFAST management is responsible for IRM assignments if such a person or organization is not agreed to. The Administrator will conform to all aspects of these Guidelines and will be subject to the oversight of the IFAST.

3.3	These guidelines do not describe the method by which IRMs are transmitted across and processed by networks. Network interworking arrangements are contained in other standards, documents, or business agreements.

  - Participation by all international carriers engaged in providing public correspondence based on wireless medium is strictly voluntary.

# 4	ASSIGNMENT PRINCIPLES

The assignment principles defined below allow all international wireless service providers the greatest possible latitude in providing seamless international roaming capabilities in ANSI-41 based systems.

  - IRMs are to be assigned, to any wireless service provider(s) meeting the criteria specified in Section 5 of these guideline, to provide roaming in ANSI-41 based networks.

4.2	IRMs are normally assigned uniquely to individual service providers. However, it is a national matter whether IRMs can/must be shared between service providers within a country. IRMs will be shared at the 5th digit level, i.e., the 4-digit IRM + the next digit (4+1 = 5-digit format) will identify the service provider. 4.3	Upon approval of the application, the IRM adinistrator will initially assign one IRM (country code/network).  An applicant(s) must provide customer forecasts to the IRM administrator when more than one code is requested.

4.4	IRMs shall be assigned to permit the most effective and efficient use of a finite resource in order to maximize the existing allocated resource inventory and to defer, as long as practical, the need to request additional MIN resources.

4.5	The IRM administrator will:

•	Assign IRMs in a fair, timely and impartial manner to any applicant that meets the criteria for assignment (Section 6).

•	Assign IRMs on a first come, first served basis from the available pool.

•	Make all assignments based on the procedures in these guidelines.

•	Treat sensitive information received from applicants as proprietary and confidential, and not to be shared with non-administrator personnel.

4.6	Information that is requested of an applicant(s) in support of an IRM application shall be uniform and kept to a minimum.

4.7	Assigned IRMs should be implemented by the assignee as soon as possible, but no later than twelve months after assignment.  If the assignee can demonstrate that an assigned IRM has not been deployed solely due to delays beyond its control, the time period can be automatically extended for up to 90 days.  The IRM assignee must, however, apply to the Administrator for such an extension. At the discretion of the administrator, three additional 90-day extensions may be granted.

4.8	Any IRMs assigned to meet the purpose of a “reserve” may be held by the applicant(s) for a period not to exceed 2 years from the date of original assignment at which time the IRM Administrator may reclaim any unused reserve IRMs

4.9	An entity(ies) which is denied an IRM assignment or extension under these guidelines has the right to appeal that decision (Section 10).

4.10	The payment of an application fee and an administrative fee is required with an application for an IRM.

4.11	As required, an IRM applicant(s) must comply with all applicable domestic regulations.

# 5	CRITERIA FOR IRM ASSIGNMENT

5.1 The assignment criteria in the following paragraphs should be considered by a potential IRM applicant(s) before submitting an IRM application and will be used by the IRM Administrator in reviewing and processing an IRM application.

  - IRMs will be assigned to any ANSI-41 based wireless network that will provide roaming with other ANSI-41 based countries, included in the NANP serving area. IRMs can also be assigned to wirless networks not based in a specific geographic country, e.g., Global Mobile Satellite Service (GMSS) networks.

# 6	Assignment Procedures

Entities requesting IRM assignments and entities already assigned one or more IRMs shall comply with the following:

6.1 IRM applicants and assignees must meet all conditions specified in these guidelines.  Copies of the guidelines may be obtained from the IRM administrator.6.2 Applicants must apply in writing to the IFAST Secretariat (contact details provided below). . The application must be accompanied by payment of the application and maintenance fees (see the IFAST website or contact the Secretariat for fee information). Applications can either be submitted electronically (credit card information required for fees) or by paper.

6.3 	If sharing of IRMs is nationally authorized/mandated and if an IRM application is for a shared implementation, the application must include the details for each entity sharing the IRM. The applications and maintenance fees are per IRM and will, consequently, be shared by the multiple applicants.

  - The IFAST Secretariat will ensure the completeness of the application, including the fees’ payment, and forward the application to the IRM Administrator for processing.

  - The IRM Administrator will process the application within 10 business days and inform the applicant of the result. If the application is denied, the IRM Administrator will provide a detailed explanation of the denial.

7.1 	The IFAST will track and monitor IRM assignments and assignment procedures to ensure that all assignments of IRMs are being used in an efficient and effective manner. Ongoing Administrator procedures that foster conservation shall include, but not be limited to, the following:

- An active reclamation  and reassignment program (detailed below) to reclaim unused or misused IRMs.

- Strict conformance with these guidelines by those assigning IRMs.

- Appropriate and timely recommended changes to these guidelines, if they are found to result in inefficient or inappropriate use or assignment of IRMs.

- Periodic specific and random audits of assignments and assignment procedures.

- An IRM Network Code recovered or returned to the administrator for reassignment will remain dormant for a period of not less than one year, from the date of return to the IRM Network Code pool, before reassignment.

7.2  Reserved IRMs may be held by the applicant for a period not to exceed 2 years from the date of original assignment at which time the IRM Administrator may reclaim the IRMs.

  - The Administrator will utilize the following 3-tiered method with regard to IRM reclamation and reassignment:

- An IRM that is reclaimed/returned, was never implemented by the original assignee, and is specifically requested by a different entity, can be assigned immediately.

- An IRM that is reclaimed/returned, whether implemented or not by the original assignee, and is specifically requested by a different entity, can be assigned following a 6-month dormant period.

- An IRM that is reclaimed/returned, whether implemented or not by the original assignee, and is not specifically requested by an entity, will remain dormant for a minimum or 6 months or until all never-assigned IRM resources have been assigned.

# 8	MAINTENANCE OF GUIDELINES

It may be necessary to modify these Guidelines periodically to meet changing and unforeseen circumstances. The need for Guidelines modification may be identified by the IRM Administrator, any entity in the international  wireless telecommunications sector or the IFAST.  When need for modification is identified by other than the IFAST, the identifying entity will submit the modification issue to the IFAST.  The forum will coordinate the modification process.  Questions or concerns regarding the maintenance of the guidelines may be directed to:

IFAST Secretariat

c/o Alliance for Telecommunications Industry Solutions (ATIS)

1200 G Street NW

Suite 500Washington, DC  20005

# 9	CONFLICT RESOLUTION

9.1       Every attempt wll be made at the time of allocation to avoid conflicts with existing IRMs.  Organizatons requesting specific IRMs should first seek the IRM availability by checking the listings found on the IFAST web site, www.ifast.org.

9.2      	If a conflict, at some later point, is determined to exisit, the following process should be followed:

- Notify the IFAST Secretariat in writing.

- Contact the organization(s) with whom the conflict exists and attempt a resolution.  Then notify the IFAST Secretariat, in writing, regarding the result of this attempt to negoatiate a resolution.

- If resolution cannot be achieved, the issue may be brought before the IFAST for a recommended solution.

# 10	APPEALS PROCESS

Disagreements may arise between the IRM Administrator and IRM applicants or assignees in the context of the administration and management of these guidelines.  In all cases, the IRM Administrator and IRM applicants/assignees will make reasonable, good faith efforts to resolve such disagreements among themselves, consistent with the Guidelines, prior to pursuing any appeal.  Appeals may include, but are not limited to, one or more of the following options:

•	The IRM applicant/assignee will have the opportunity to resubmit the matter to the IFAST for reconsideration with or without additional input. The appeal should be submitted in writing to the IFAST Secretariat. The Secretariat will include the appeal on the agenda of the subsequent IFAST meeting, for resolution.

•	Guidelines interpretation/clarification questions may be referred to the IFAST for resolution.  Unless otherwise mutually agreed to by the parties, these questions will be submitted in a generic manner protecting the identity of the appellant.

•	The applicant/assignee may pursue the disagreement with the appropriate governmental/regulatory body.

Reports on any resolution resulting from the above options, the content of which will be mutually agreed upon by the involved parties, will be kept on file by the Administrator. At a minimum, the report will contain the final disposition of the appeal; e.g., whether or not an IRM was assigned.

North American Numbering Plan, the numbering plan of 18 North American Countries.

An open international industry forum, sponsored by the Alliance for Telecommunications Industry Solutions (ATIS).