### IFAST-15

## JANUARY 25, 2001

CROSS TECHNOLOGY ROAMING STANDARDS

# DOUGLAS ROLLENDER

##### LUCENT TECHNOLOGIES

#### OVERVIEW

- CROSS TECHNOLOGY ROAMING (XTR) STANDARDS:  PAST, PRESENT, FUTURE

- J-STD-O38 INTRODUCTION AND REVIEW

- FUTURE REQUIREMENTS

###### XTR STANDARDS

- IS-129

Interworking/Interoperability Between DCS1900 and

IS-41 Based MAPs for 1800 MHz Personal Communications Systems (1996)

- PCS1900 (GSM)  ANSI-553 (Analog)

- Dual Mode Phone and SIM Card

- PN-4857 (to be J-STD-038)

Network Interworking Between GSM MAP and

ANSI-41 MAP (est. 2001)

- GSM     ANSI-136 (TDMA)

- Dual Mode Phone and SIM Card

J-STD-038 XTR SERVICE DESCRIPTIONS

- AUTHENTICATION

- CALL FORWARDING

- UNCONDITIONAL

- BUSY

- NO ANSWER / NO REPLY / NOT REACHABLE

- DEFAULT

- OPTIONAL ROUTING FOR LATE CALL FORWARDING

- CALL WAITING

- 3-WAY CALLING / MULTI-PARTY

- CALLING # / LINE PRESENTATION

- CALL BARRING & OPERATOR DETERMINED BARRING

- SHORT MESSAGE TELESERVICE SUPPORT

- MESSAGE WAITING NOTIFICATION

### J-STD-038 XTR NETWORK FEATURES

- LOCATION REGISTRATION

- FAULT RECOVERY PROCEDURES

- SUBSCRIBER DATA MANAGEMENT

- AUTOMATIC CALL DELIVERY

HLR  VLR INTERFACES

MSC  MSC INTERFACE

MC  MSC INTERFACE

MC  MC INTERFACE

XTR STANDARDS IN DEVELOPMENT (cont’d.)

- PN-4926

IIF Enhancements to Support Two Way Roaming

- Modify J-STD-038

- CDMA  GSM

- SIM Card

- IIF with Service Orders (like GAIT)

- Two Way Authentication

IIF SUBSCRIBER IDENTIFICATION  INFORMATION

- International Mobile Subscriber Identity (IMSI)

- Mobile Identification Number (MIN)

- Electronic Serial Number (ESN)

- Terminal type

SUBSCRIBER AUTHENTICATION INFORMATION

- Ki (GSM subscriber authentication key)

- Triplets or groups of Kc (cipher key), CKSN (cipher key sequence number), and SRES (signed response) for GSM based authentication and ciphering

- A-key (AMPS/ANSI-136 authentication key)

- SSD-A (AMPS/ANSI-136 shared secret data for authentication)

- SSD-B (AMPS/ANSI-136 shared secret data used for generated signaling message encryption (SME) and voice privacy (VP) masks)

REGISTRATION IN GSM NETWORK – TWO WAY AUTHENTICATION

XTR STANDARDS IN DEVELOPMENT

- PN-4925 (to be IS-868)

TIA/EIA-41 Enhancement to Support SIM Roaming to GSM (2001)

- CDMA  GSM

- Dual Mode SIM Card

- PN-4927

IIF Enhancements to Support One Way SIM Roaming to GSM

- Modify J-STD-038

- CDMA  GSM

- SIM Card

- IIF Without Service Orders

- One Way Authentication

ONE WAY AUTHENTICATION for ONE WAY ROAMING

- CDMA  GSM ONLY

- SIM CARD EXECUTES ANSI-41 AUTHENTICATION (CAVE) IN GSM FOREIGN MODE

- IIF HAS NO PERMANENT SUBSCRIBER DATA OR AUTHENTICATON CENTER

- IIF GETS ESN FROM HLR FOR AUTHENTICATION  (NEW ANSI-41 MESSAGE)

REGISTRATION IN FOREIGN GSM NETWORK WITH ONE WAY AUTHENTICATION

MS                  GSM MSC/VLR                   IIF            ANSI-41 HLR/AC    ANSI-41 MSC/VLR

Location_Area_Update (IMSI)

UPDATE_LOCATION [IMSI]

Location_Area_Update ack

UPDATE_LOCATION ack

INSERT_SUBSCRIBER_DATA

INSERT_SUBSCRIBER_DATA ack

Authenticate ack (SRES)

Authenticate (RAND)

SEND_AUTHENTICATION_INFO [IMSI]

SEND_AUTHENTICATION_INFO ack

[ AuthenticationSetList (RAND, SRES, Kc)]

ESNQUERY [MSID]

esnquery [ESN]

ASREPORT

asreport

ASRRT

AUTHREQ [MSID, ESN]

REGCANC [MSID, ESN]

regcanc

regnot [profile]

REGNOT [MSID, ESN]

authreq [RANDU, AUTHU, SSD]

New message for IIF.

This mapping depends on the security mechanism.

PROPOSED CROSS TECHNOLOGY ROAMING STANDARDS

- J-STD-038 Rev.A (2001)

- GAIT (GSM+TDMA)  GPRS

- CDMA GSM (One Way Authentication)

- CDMA  GSM (Two Way Auth.)

FURTHER CROSS TECHNOLOGY ROAMING STANDARDS REQUIREMENTS and IMPLEMENTATION SUPPORT for ANSI-41 NETWORKS

- GGRF

- GAIT

- G95

- iDEN

- TETRA

- CDG

- UWCC

- IFAST